Intro to System-on-Chip Design Course

LAB 9

Programming an SoC using C Language

Issue 1.0

- Contents

1	Introduction	1

1.1	Lab overview	1

1.1.1	Software programming:	1

1.1.2	Demonstrate the SoC:	1

2	Learning Objectives	1

3	Requirements	2

4	Provided files	2

5	Software	3

5.1	Program procedure	3

5.2	An example of the demo:	4

6	Extension work	4

6.1	Extra tasks for this lab:	4

# Introduction

## Lab overview

In the previous labs, assembly language was used to program the SoC at low-level, which gave better control over the hardware and helped us understand the hardware mechanism at register transfer level (RTL).

In this lab, C language will be used to program the SoC at a higher level, whereby compilers, software libraries, and tools can be used to hide the complexity of low-level management. This means that we can make more complicated applications in a more efficient way.

One can always mix C and assembly code in one project, or even in one file.

Hardware implementation has been completed in the previous labs; the following labs will focus on software. The work in this lab includes:

### Software programming:

- Program the Cortex-M0 processor and control the peripherals using C language.

### Demonstrate the SoC:

- Use the timer interrupt to implement a counting up counter (counting from 0 to 9) and display the value to the VGA display (same as previous lab but using C programming).

- Use the UART interrupt to send characters to a PC or laptop (same as previous lab but using C programming).

- Input from 8-bit switch and output to LEDs

# Learning Objectives

- Implement a timer interrupt handler and UART interrupt handler in a high-level language such as C.

- Modify a C program to enable the timer and UART interrupts.

- Modify a C program to set the interrupt priority of the timer and UART interrupts.

- Modify a C program to initialise the timer to generate an interrupt at regular interval.

# Requirements

This lab requires the following hardware and software:

- Hardware:

  - Diligent BASYS 3 FPGA board connected to computer via MicroUSB cable. A constraints file for this board is also provided.

  - VGA-compliant monitor and VGA cable to connect your board

- Software

  - Xilinx Vivado

  - Keil uVision

  - TeraTerm

# Provided files

You will need the files from the previous labs along with the following files which are provided with this Lab:

| File name | Description |
| --- | --- |
| cm0dsasm.s | The assembly file that includes interrupt vectors and part of the interrupt service routines |
| main.c | The C code that contains the main program and part of the interrupt service routines |

# Software

## Program procedure

The assembly code in cm0dsasm.s should perform the following:

- Initialize the interrupt vector.

- Define heap and stacks.

- Reset handler.

  - Branch to the main code in main.c.

- Timer handler

  - Push registers (e.g., R1 – R4) to the stack.

  - Branch to the timer interrupt service routine in main.c.

  - Pop registers from the stack.

- UART handler

  - Push registers (e.g., R1 – R4) to the stack.

  - Branch to the UART interrupt service routine in main.c.

  - Pop registers from the stack.

The C code in main.c should perform the following:

- Main program

  - Set the timer interrupt priority to 0x00 (higher).

  - Set the UART interrupt priority to 0x40 (lower).

  - Enable interrupts for the timer and UART.

  - Reset the 7-segment display.

  - Initialize the timer to generate an interrupt every second.

    - Write the load value register, e.g., 50,000,000.

    - Set prescaler, e.g., 1x or 16x.

    - Change the operation mode to load mode.

  - Start the timer.

  - Repeat the following:

    - Change the GPIO to read mode.

    - Read from the switches using the GPIO.

    - Change the GPIO to write mode.

    - Write to the LEDs.

- Timer interrupt handler

  - Clear the timer interrupt request.

  - Increment the counter.

  - Display the counter to the VGA text region.

  - Disable the timer interrupt if the counter reaches 9.

- UART interrupt handler

  - Read from the UART (from the keyboard).

  - Write to the UART (to the terminal window).

## An example of the demo:

Figure 1:VGA Demo

# Extension work

## Extra tasks for this lab:

- Optimize the code in both operational speed and code density. You can use:

  - Embedded assembly code in C code

  - Embedded C code in assembly code

- Evaluate the coding efficiency between handwritten assembly code and assembly generated by the compiler.